VLANs and Secure Switch Configuration

Introduction.

This activity involves configuring network devices and implementing Layer 2 security features to ensure secure and efficient communication within a network. The exercise involves setting up routers, switches, PCs, configuring VLANs, and securing unused switchports.

Part 1: Configure the Network Devices.

Step 1: Cable the network.

Step 2: Configure R1.

Verify the running-configuration on R1

Step 3: Configure and verify basic switch settings.

Configure the hostname for switches S1 and S2.

Prevent unwanted DNS lookups on both switches.

Configure interface descriptions for the ports that are in use in S1 and S2.

Set the default-gateway for the Management VLAN to 192.168.10.1 on both switches.

Part 2: Configure VLANs on Switches.

Configure VLAN 10.

Configure the SVI for VLAN 10.

Configure VLAN 333 with the name Native on S1 and S2.

Part 3: Configure Switch Security.

Step 1: Implement 802.1Q trunking.

1.On both switches, configure trunking on F0/1 to use VLAN 333 as the native VLAN.

2. Verify that trunking is configured on both switches.

3.Disable DTP negotiation on F0/1 on S1 and S2.

4. Verify with the show interfaces command.

Step 2: Configure access ports.

Step 3: Secure and disable unused switchports.

Show interface status.

Step 4: Document and implement port security features.

Port security interface f0/6 on S1 and enable it.

| Default Port Security Configuration Feature | Default Setting |
| --- | --- |
| Port Security | Enabled |
| Maximum number of MAC addresses | 3 |
| Violation Mode | Restrict |
| Aging Time | 60mins |
| Aging Type | Inactivity |
| Secure Static Address Aging Sticky MAC Address | Disabled 0 |

Verify port security on S1 F0/6.

4. Enable port security for F0/18 on S2 and show security interface.

Step 5: Implement DHCP snooping security.

Verify DHCP Snooping on S2.

From the command prompt on PC-B, release and then renew the IP address.

Step 6: Implement PortFast and BPDU guard on S1 and S2.

Verify that BPDU guard and PortFast are enabled on the appropriate ports.

show spanning-tree interface f0/6 detail

Step 7: Verify end-to-end connectivity.

CONCLUSION.

In conclusion, I have gained practical experience configuring VLANs. I learned how to implement VLANs, secure access ports, and manage unused switchports, which are vital for network segmentation and security. One of the challenges I faced was improper configuration of the network, which resulted to getting errors as the output. I learnt how to troubleshoot the problems which improved my troubleshooting skills and deepened my understanding of secure network design.